Ball & Beam — PID Gain Derivation

github.com/smeschke/ball_and_beam

# 1. Overview

A ball rolls on a beam tilted by a stepper motor via a cam mechanism. A camera detects ball position (normalized 0–1 across the beam). A PC runs a PID controller and sends signed velocity commands (steps/sec) over serial to an Arduino, which drives the stepper. The goal is to hold the ball at a target position.

Because the output unit is steps/sec rather than beam angle, the PID gains are much larger than textbook values. This document derives them from first principles.

# 2. Physical Parameters

| Parameter | Value |
| --- | --- |
| Beam active length | 21 in = 0.533 m |
| Stepper resolution | 25,000 steps / revolution |
| Cam full stroke | 12,500 steps → full left to full right |
| Max beam tilt | ~5.5° = 0.096 rad (avg of 6.4° / 4.7° measured by phone) |
| Angle per step | 0.096 / 12,500 = 7.75 × 10⁻⁶ rad/step |
| Ball | 22 mm ceramic bearing (solid sphere) |
| MAX_VEL | 8,000 steps/sec |

# 3. Plant Model

## 3.1  Ball dynamics

For a solid sphere rolling without slip on an inclined beam (small angle approximation):

pos_ddot  =  (5g/7) × θ / beam_length

=  (5 × 9.81 / 7) × θ / 0.533

=  13.15 × θ     [normalized/s²  per  radian]

## 3.2  Plant gain K

The controller commands velocity (steps/sec). Motor position is the integral of velocity, and beam angle is proportional to motor position. This makes the plant a double integrator — velocity → position — with effective gain K:

K  =  13.15 × (0.096 / 12,500)

=  13.15 × 7.75 × 10⁻⁶

=  1.02 × 10⁻⁴   [normalized/s²  per  step/s]

Sanity check: at 1,000 steps/sec sustained for 1 second, expected ball displacement ≈ ½ × K × v × t² = ½ × 1.02×10⁻⁴ × 1000 × 1 = 0.051, or about 5% of beam length. That is physically reasonable.

# 4. PID Gain Derivation

With a PD controller on a double integrator G(s) = K/s², the closed-loop characteristic equation is:

s²  +  K·Kd·s  +  K·Kp  =  0

Matching to the standard second-order form  s² + 2ζωn·s + ωn²  gives:

Kp  =  ωn²   / K

Kd  =  2·ζ·ωn / K

For critical damping (ζ = 1, no overshoot) the settling time is t_s ≈ 4/ωn, so:

ωn  =  4 / t_settle

| Desired settling time | ωn (rad/s) | Kp | Kd |
| --- | --- | --- | --- |
| 1.5 s (fast) | 2.67 | 69,900 | 52,400 |
| 2.0 s (recommended) | 2.00 | 39,200 | 39,200 |
| 3.0 s (conservative) | 1.33 | 17,400 | 26,100 |

Note: at ζ = 1 and ωn = 2, Kp = Kd because ωn² = 2·ωn when ωn = 2. For other values of ωn, Kd > Kp.

# 5. Lead-Lag Compensator — Critical Role

The system uses a first-order lead-lag filter on the PID output, implemented via the bilinear (Tustin) transform:

H(s)  =  (τ_lead · s + 1) / (τ_lag · s + 1)

Current tuning: τ_lead = 500 ms,  τ_lag = 150 ms.  Since τ_lead > τ_lag, this is a phase-lead compensator — it amplifies mid-to-high frequencies and adds phase advance. In practice it acts as a strong additional derivative term on top of the PID's own Kd.

This is why the experimentally working gains (Kp = 1000, Kd = 5830) are roughly 30–40× lower than the theoretical values above. The lead-lag filter is supplying most of the derivative action. The PID gains and the lead-lag must be tuned together — changing one without the other will destabilize the system.

Practical consequence: if the lead-lag is disabled (L key), the system will oscillate and run away even with Kd = 5830, because the raw PD gains are insufficient without the compensator. This is the expected behavior, not a bug.

| τ_lead > τ_lag | Phase lead — faster response, more derivative action, can cause overshoot |
| --- | --- |
| τ_lead < τ_lag | Phase lag — smoothing / low-pass, reduces noise sensitivity |
| τ_lead = τ_lag | Unity gain — filter has no effect |

To reduce overshoot from disturbances: lower τ_lead toward τ_lag. To reduce noise-driven chatter in the d_term: increase τ_lag. These are the primary tuning levers once Kp and Kd are set.

Physical measurements: beam 21", stepper 25k steps/rev, cam stroke 12,500 steps, tilt ≈5.5°, 22mm ceramic bearing.